Challenges:

1-install library of sensor MAX30100.

2-uses of each sensor.

3-pins of 21 ,22 of I2C connection  between MAX30100 and LCD has problem in connection but  we try

the chip of MAX30100 with serial monitor because if we connect lcd not I2C with the max that use I2C the  pins will not be enough

4- we connect key pad in our project but it uses 8 pins so then we face lack of pins to connect the other components .so we remove it to be able to connect the other components

Setup Phase:

- Start Serial Communication:

  - The Serial.begin(115200); command initializes serial communication at a baud rate of 115200. This is used for debugging and displaying messages in the Serial Monitor.

- Initialize I2C and LCD:

  - Wire.begin(21, 22); sets up I2C communication with specific GPIO pins for SDA and SCL.

  - lcd.begin(16, 2); initializes the LCD with 16 columns and 2 rows.

  - lcd.backlight(); turns on the backlight of the LCD for better visibility.

- Initialize Sensors:

  - sensors.begin(); initializes the DS18B20 temperature sensor.

  - The Pulse Oximeter (MAX30100) is initialized with pox.begin(). If it fails, the program halts with an error message.

- Setup Pins:

  - Define and configure GPIO pins for IR sensor, ECG sensor, pressure sensor, LEDs, buzzer, and servo motor.

  - Set LED and buzzer pins to OUTPUT mode and initialize the servo motor to 0 degrees.

- Configure Pulse Oximeter:

  - Set the IR LED current and specify a callback function to handle heartbeat detection.

2. Main Process

Loop Phase:

- Update Pulse Oximeter:

  - pox.update(); checks and updates the Pulse Oximeter readings for SpO2 and heart rate.

- Read IR Sensor:

  - bool humanDetected = digitalRead(IR_SENSOR_PIN) == LOW; reads the IR sensor to determine if a human is present.

- If Human Detected:

  - Timing Control:

    - The millis() function provides the current time in milliseconds. It is used to manage the timing of sensor data updates without blocking the program.

    - Update sensor readings every 5 seconds using the previousMillis and interval variables.

  - Display Sensor Data:

    - Depending on the mode (displayMode), the program updates the LCD with data from one of the sensors:

      - Mode 0: Displays temperature from DS18B20.

      - Mode 1: Displays ECG values.

      - Mode 2: Displays pressure values.

      - Mode 3: Displays SpO2 and heart rate from MAX30100.

    - The servo.write() function moves the servo to different angles based on the mode.

- If No Human Detected:

  - Display Message:

    - Show a message "No human detected" on the LCD indicating the system is in standby mode.

  - Turn Off Alerts:

    - Ensure the green LED, red LED, and buzzer are turned off.

  - Reset Servo:

    - Move the servo back to its initial position (0 degrees).

3. Sensor Classification

Classification Functions:

- Temperature (classifyTemperature(float temperature)):

  - Categorizes temperature readings as "Low", "Normal", or "High" based on predefined thresholds.

- ECG (classifyECG(float ecgValue)):

  - Determines whether ECG values are "Normal" or "High".

- Pressure (classifyPressure(float pressureValue)):

  - Classifies pressure readings as "Normal" or "High".

- SpO2 (classifySpO2(float spO2Value)):

  - Indicates if SpO2 levels are "Low" or "Normal".

4. Alerts and Notifications

- Trigger Alerts (triggerAlert()):

  - Activates the red LED, buzzer, and moves the servo to 180 degrees to signal an alert condition.

  - The alert persists for 2 minutes before the servo returns to 0 degrees, and the buzzer is turned off.

- Heartbeat Detection (onBeatDetected()):

  - A callback function that prints a message to the Serial Monitor whenever a heartbeat is detected by the Pulse Oximeter.